# CZO Metadata Worksheet

| Data File Name | CZO_DOE Spatial CO2 Survey 2016 |
| --- | --- |
| Record Period | 01/06/2016 to 10/06/2016 |
| Descriptive Title | Susquehanna Shale Hills Critical Zone Observatory & DOE Spatial CO2 Survey Data |
| Update Frequency | Yearly |
| Abstract | Soil CO2 surveys were conducted at three soil collars distributed at 50 sites across the Shale Hills Susquehanna Critical Zone Observatory Stream. Soil CO2 surveys were conducted approximately biweekly from April-September and monthly in January-April and September-October. |
| Investigator Contact Info | - Dr. David M. Eissenstat, Professor of Woody Plant Physiology, The Pennsylvania State University, 201 Forest Resources Building, University Park, PA 16802, (814)-863-3371 dme9@psu.edu |
| Data Value Descriptions | - COL1.label=File Name value=General, format= data collector initials-date-site number- soil collar letter - COL2.label=Date_Name, value=Time Stamp, UTCOffset=-5, TimeZone=EST, format=MM/dd/yyyy HH:mm - COL3.label=Exp_Flux, value= Soil CO2 flux, units = umol/m2/s, Method = LICOR 8100A Soil CO2 flux chamber, SampleMedium =Soil, ValueType=Sampled, DataType=Average - COL4.label=V4_Mean, value= Soil moisture content, units = volume water/volume soil, Method = Theta Probe, SampleMedium =Surface water (5 cm depth), ValueType=Sampled, DataType=Average - COL5.label=Tcham_Mean, value= Chamber temperature, units = Degrees Celsius, Method = LICOR 8100A Soil CO2 flux chamber, SampleMedium = LICOR 8100A Soil CO2 flux chamber, ValueType=Sampled, DataType=Average - COL6.label=Pressure_Mean, value= Chamber pressure, units = KPa, Method = LICOR 8100A Soil CO2 flux chamber, SampleMedium = LICOR 8100A Soil CO2 flux chamber, ValueType=Sampled, DataType=Average - COL7.label=H20_Mean, value= Chamber water, units = mol water/mol gas in chamber, method = LICOR 8100A Soil CO2 flux chamber, SampleMedium = LICOR 8100A Soil CO2 flux chamber, ValueType=Sampled, DataType=Average |
| Keywords | - Soil gas, soil respiration, soil CO2 efflux, surface soil water |
| Methods | Measurements taken with LICOR 8100A Soil CO2 flux chamber. Excel files from each collection point were run and checked for quality in SoilFluxPro. Values with negative soil efflux were removed. Values with extreme starting CO2 data were checked. Measurements with bad CO2 efflux over time curves were either sampled where their curves were reasonable or removed from the set. Chamber temperature, pressure, and H2O ranges and means were examined over time to ensure no unrealistic means were used. |
| Citation | The following acknowledgment should accompany any publication or citation of these data: Logistical support and/or data were provided by the NSF-supported Shale Hills Susquehanna Critical Zone Observatory. |
| Publications | none |
| Data Use Notes | The user of Shale Hills Susquehanna CZO data agrees to provide proper acknowledgment with each usage of the data. Citation of the name(s) of the investigator(s) responsible for the data set, in addition to the generic statement above, constitutes proper acknowledgment. Author(s) (including Shale Hills Susquehanna CZO investigators) of published material that makes use of previously unpublished Shale Hills Susquehanna CZO data agree to provide the Shale Hills Susquehanna CZO data manager with four (4) copies (preferably reprints) of that material for binding as soon as it becomes available. The user of Shale Hills Susquehanna CZO data agrees not to resell or redistribute shared data. The user of these data should be aware that, while efforts have been taken to ensure that these data are of the highest quality, there is no guarantee of perfection for the data contained herein and the possibility of errors exists. These data are defined as either public or private, such that a password may be required for access. |